Date:

EXPERIMENT: 13

AIM: Learn to utilize Wi-Fi Module on ESP32 and implement codes to : i. scan Wi-Fi networks ii. Set up a simple Wi-Fi web server to blink an LED from the web and iii. set up a Wi-Fi access point and provide a web server on it.

OBJECTIVES:

- Implement code to scan Wi-Fi networks

- Implement code to set up a simple Wi-Fi web server to blink an LED from the web

- Implement code to set up a Wi-Fi access point and provide a web server on it

COMPONENTS:

CONNECTION DIAGRAM:

CODES:

13.1:

#include "WiFi.h" void setup()

{

Serial.begin(115200);

WiFi.mode(WIFI_STA);     WiFi.disconnect();     delay(100);

Serial.println("Setup done");

}

void loop()

{

Serial.println("scan start");     int n = WiFi.scanNetworks();     Serial.println("scan done");     if (n == 0)

{

Serial.println("no networks found");

}     else

{

Serial.print(n);

Serial.println(" networks found");         for (int i = 0; i < n; ++i)

{

// Print SSID and RSSI for each network found Serial.print(i + 1);        Serial.print(": ");

Serial.print(WiFi.SSID(i));

Serial.print(" (");

Serial.print(WiFi.RSSI(i));

Serial.print(")");

Serial.println((WiFi.encryptionType(i) == WIFI_AUTH_OPEN) ? " " : "*");             delay(10);

}

}

Serial.println("");     delay(5000);

}

13.2:

#include<WiFi.h>

const char* ssid = "DEVAL";

const char* password = "12345678";

WiFiServer server(80);

void setup()

{

Serial.begin(115200);

pinMode(2, OUTPUT);

delay(10);

Serial.println();

Serial.println();

Serial.print("Connecting TO ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED){

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi Connected.");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

server.begin();

}

int value = 0;

void loop() {

WiFiClient client = server.available();

if(client) {

Serial.println("New Client.");

String currentLine = "";

while (client.connected()) {

if (client.available()) {

char c = client.read();

Serial.write(c);

if(c =='\n') {

if(currentLine.length() ==0) {

client.println("HTTP/1.1 200 OK");

client.println("Content-type:text/html");

client.println();

client.print("Click <a href=\"/H\"here</a> to turn the LED on pin 2 on.<br>");

client.print("Click <a href=\"/L\"here</a> to turn the LED on pin 2 off.<br>");

client.println();

break;

}else{

currentLine = "";

}

}else if (c != '\r'){

currentLine += c;

}

if(currentLine.endsWith("GET /H")) {

digitalWrite(2, HIGH);

}

if (currentLine.endsWith("GET /L")) {

digitalWrite(2, LOW);

}

}

}

client.stop();

Serial.println("Client Disconnected");

}

}

13.3:

#include <WiFi.h>

#include <WiFiClient.h>

#include <WiFiAP.h>

#define LED_BUILTIN 2   // Led Signal Pin

// Set these to your wifi access point credentials.

const char *ssid = "yourAP";

const char *password = "yourPassword";

WiFiServer server(80);

void setup() {

pinMode(LED_BUILTIN, OUTPUT);

Serial.begin(115200);

Serial.println();

Serial.println("Configuring access point...");

// You can remove the password parameter if you want the AP to be open.

WiFi.softAP(ssid, password);

IPAddress myIP = WiFi.softAPIP();

Serial.print("AP IP address: ");

Serial.println(myIP);

server.begin();

Serial.println("Server started");

}

void loop() {

WiFiClient client = server.available();   // listen for incoming clients

if (client) {                             // if you get a client,

Serial.println("New Client.");           // print a message out the serial port

String currentLine = "";                // make a String to hold incoming data from the client

while (client.connected()) {            // loop while the client's connected

if (client.available()) {             // if there's bytes to read from the client,

char c = client.read();             // read a byte, then

Serial.write(c);                    // print it out the serial monitor

if (c == '\n') {                    // if the byte is a newline character

// if the current line is blank, you got two newline characters in a row.

// that's the end of the client HTTP request, so send a response:

if (currentLine.length() == 0) {

// HTTP headers always start with a response code (e.g. HTTP/1.1 200 OK)

// and a content-type so the client knows what's coming, then a blank line:

client.println("HTTP/1.1 200 OK");

client.println("Content-type:text/html");

client.println();

// the content of the HTTP response follows the header:

client.print("Click <a href=\"/H\">here</a> to turn ON the LED.<br>");

client.print("Click <a href=\"/L\">here</a> to turn OFF the LED.<br>");

// The HTTP response ends with another blank line:

client.println();

// break out of the while loop:

break;

} else {    // if you got a newline, then clear currentLine:

currentLine = "";

}

} else if (c != '\r') {  // if you got anything else but a carriage return character,

currentLine += c;      // add it to the end of the currentLine

}

// Check to see if the client request was "GET /H" or "GET /L":

if (currentLine.endsWith("GET /H")) {

digitalWrite(LED_BUILTIN, HIGH);               // GET /H turns the LED on

}

if (currentLine.endsWith("GET /L")) {

digitalWrite(LED_BUILTIN, LOW);                // GET /L turns the LED off

}

}

}

// close the connection:

client.stop();

Serial.println("Client Disconnected.");

}

}.

OUTPUTS:

1.

2.

3.

OBSERVATIONS:

CONCLUSION:

DRIVE LINK OF VIDEO:

- https://drive.google.com/file/d/1BijVP344BNwNPi5kYSOlj9Vl82d9H2gu/view?usp=drivesdk

- https://drive.google.com/file/d/1uRaXNCl9b9Tlpnn3KL3dx1GNNeqDVIic/view?usp=drivesdk

- https://drive.google.com/file/d/1A_H-JYHqqUVvbB9ogBIpG60f0YEKkbiR/view?usp=drivesdk

SUBMITTED BY:

- 23CS041- DHRUV LOKADIYA

- 23CS045- MITUL MISTRY

- 23CS046- KATHAN MODH